Table of contents

1	Features	1

2	Installation	2

3	General	2

3.1	Coordinate system translation	2

4	AutoCAD (pov.lsp)	2

4.1	Draw/Create	2

4.1.1	Command syntax	2

4.2	Lights, camera, action	4

4.2.1	Animation	4

4.2.2	Command syntax	4

4.3	Modify	5

4.3.1	Command syntax	5

4.4	Other	5

5	AutoCAD2POV.exe	5

5.1	Files	6

5.2	Blocks	6

5.3	Layers	6

5.4	Animation	6

6	Contact the author	6

7	In the future	6

8	Known issues	6

# Features

AutoCAD2POV converts DXF files to POV.  However, this is not just another dxf2pov.  One of the major shortcomings of all other DXF converters is their inability to convert AutoCAD solid models.  AutoCAD2POV will not convert pre-existing solid models, but it will convert solid models created with the aid of the AutoCAD2POV LISP file.  The AutoCAD2POV executable will convert even pre-existing DXF files with all of the features listed below, just without support for solid models, camera and target points/paths and lighting.  The LISP file and the EXE file combine forces to provide you, the user, with the following outstanding features (insert infomercial here):

- Full support for corresponding CSG commands

- Full support for 3Dfaces and 3D meshes

- Creates textures from layer definitions and applies them to objects

- Support for Nathan Kopp’s patched POV (uvPOV)

- Support for blocks and nested blocks (composed of valid 3D objects)

- Insert lights, cameras and target (look_at) points and/or paths

- Intuitive coordinate system translation

# Installation

After installing the executable you will need to either add the program path to your AutoCAD search path or copy the following files to your AutoCAD Support directory: pov.lsp, povconvert.mnl, povconvert.mnu.  To load the LISP file into AutoCAD type (load “pov.lsp”) at the command prompt.  This will also load the menu and the toolbar.  The LISP file must be loaded every time you enter a drawing, so you may consider appending (load “pov.lsp”) to your acad.lsp file.

# General

AutoCAD2POV is a utility for use by those who wish to use AutoCAD to model for POV-Ray.  The EXE file should work with all DXF files, even without the use of the LISP file (see Section 1).  The LISP file should work on all AutoCAD versions (solid modeling features are obviously limited to those versions with such).

## Coordinate system translation

Let me explain the coordinate system used in the conversion process.  Of primary concern is the fact that AutoCAD uses a right-handed system while POV-Ray uses the left-handed system.  Of secondary concern is that most people who use AutoCAD draw the plan view in the X-Y plane, whereas the X-Y plane in POV-Ray traditionally defines the front view.  Therefore to avoid the confusion of both translating between coordinate systems and drafting mentalities, I devised the RUA coordinate system.  RUA would be Right, Up, Away.  Translated into AutoCAD this would be X(R),Z(U),Y(A).  In POV-Ray it would be X(R),Y(U),Z(A).  As you can see, this basically involves transposing the Y and Z vectors.  You may notice however, that this inverts rotation angles.  I decided the compromise was worth the simplicity and intuitiveness of the conversion.  The conversion program inverts any rotation angles in the process.

# AutoCAD (pov.lsp)

The AutoCAD end of things uses a small LISP file and a menu and toolbar.  The LISP file includes replacements for AutoCAD commands that correspond to POV CSG operations.  For proper conversion, it is imperative that the substitute commands be used when available.  The substitute commands also include those for modifying solid primitives.  Most substitute commands are standard AutoCAD commands prefixed with a “p”.  These are enumerated below.  They follow the general nature of the POV modeling sequence (e.g. protate rotates objects around the axis, so you would create a box at 0,0,0 then rotate it, then translate it).

Note:  All commands can be accessed via the menu or the toolbar (and it’s probably easier that way).

## Draw/Create

### Command syntax

psphere

Substitute command for sphere

Usage:

Command:psphere

Center of sphere: select point or enter coordinates

Radius of sphere: select distance or enter radius

Sphere complete

pbox

Substitute command for box

Usage:

Command:pbox

Corner of box: select point or enter coordinates

Other corner: select point or enter coordinates

If the two points are on the same R-A plane, pbox continues:

Height:  select distance or enter height

Box complete

pcyl

Substitute command for cylinder

Command:pcyl

Center of cylinder: select point or enter coordinates

Radius: select distance or enter radius

Center of other end/<Height>: select distance for height or type C

If height is entered, the cylinder is drawn perpendicular to the R-A plane

If “C” is entered, pcyl continues:

Center of other end: select point or enter coordinates

Cylinder complete

pcone

Substitute command for cone

Command:pcone

Center of base: select point or enter coordinates

Radius of base: select distance or enter radius

Center of other end/<Height>: select distance for height or type C

If height is entered, the cone is drawn perpendicular to the R-A plane

If “C” is entered, pcone continues:

Center of other end: select point or enter coordinates

Cone complete

ptorus

Substitute command for torus

Note: Torus is drawn with center at 0, 0, 0.

Command:ptorus

Radius of torus: select distance or enter radius

Radius of tube: select distance or enter radius

Torus complete

ppline

Draws a pline in the correct plane for the pextrude or prevolve commands (below)

Command:ppline

Prism or Lathe object? Enter P(rism) or L(athe)

Depending on which object is selected, the drawing plane changes and ppline continues:

Pick first point: select point or enter coordinates

Pick point: select next point or enter coordinates

…

Pick point: select next point, enter coordinates or press enter to end

Ppline complete

Ppline returns to the previous drawing plane

pspline

Draws a spline in the correct plane for the pextrude or prevolve commands (below)

See usage for ppline

pextrude

Creates a (POV) Prism object from a pline or spline created with the ppline or pspline command

Command:pextrude

Select closed polyline to extrude: select previously drawn ppline or pspline

Height of extrusion: select distance or enter height

If an invalid object is selected pextrude aborts

Prism complete

prevolve

Creates a (POV) Lathe object from a pline or spline created with the ppline or pspline command

Command:prevolve

NOTE--Polyline will be revolved around the UP-Axis at 0,0,0

Select closed polyline to revolve: select previously drawn ppline or pspline

If an invalid object is selected prevolve aborts

Lathe complete

ptext

Substitute command for text

Command:ptext

Start point: select point or enter coordinates

Text thickness: select distance or enter thickness

String of text: Type text to be drawn

Note: Text is drawn on the R-U plane and will not have thickness when viewed in AutoCAD.  The default font is Arial.

Text object complete

## Lights, camera, action

### Animation

There are three possible scenarios in AutoCAD2POV for scene animation.  1) A moving camera path and a moving target path, 2) a moving camera path with a stationary target point, and 3) a stationary camera point with a moving target path.  If a camera path is used with either a stationary or moving target, it should be defined first (before the target).  The number of points in the camera path will define how many frames are in the final animation.  After defining the camera path, if a target path is used, the tarpath command will terminate automatically after the correct number of points have been defined.  If no camera path has been defined, tarpath will not terminate automatically.  Tarpath will then define how many frames will be in the animation scene.  It is very important that the number of points in the camera path is the same as the number of points in the target path (scenario #1) for the animation to render correctly.  Svpath will only be available after a camera path has been defined.

### Command syntax

camera

Creates a camera point and a target (look_at) point

Command:camera

Camera location: select point or enter coordinates

Camera look at: select point or enter coordinates

Camera type <Standard>: type valid POV camera type or <ENTER> for standard

Sky vector angle (90 degrees = up): select or type the angle of the sky vector—90° is a vertical camera, 30° is tilted to the right, -30° is tilted to the left, etc…

Camera angle/<Ignore>: select the include angle of the camera or <ENTER> for none (see POV camera documentation for specifics)

Camera point complete—a point will be drawn at both the location and the look_at vectors

campath

Creates an animated camera path. When Auto mode is toggled on, campath maintains a constant distance (in both the X-Y and Y-Z planes) from the last selected point based on the Interval settings and the selected angle of movement.  With Auto mode toggled off, it is left to the user to select the distance, angle, etc.  Auto mode is always on when the command starts.  The number of points in the polyline will determine how many frames are in the final animation. See section

Command:campath

Select/<Start point of path>: type S(elect) or select start point for camera path

If Select is chosen, campath expects a 3Dpolyline or polyline to be chosen for the camera path

If a point is selected, campath continues:

Auto/Intervals/<Angle of movement>: type A(uto), I(ntervals), or select or type the angle of camera movement

If A(uto) is chosen, this will toggle Auto mode and campath continues:

Auto/<Next point>: select next point or enter coordinates

If I(ntervals) is chosen, campath continues:

X-Y distance: select or enter the distance in the X-Y plane to move each time

Z-distance: select or enter the distance in the Y-Z plane to move each time

Angle of movement: select or type the angle of camera movement

This continues in a loop until the user types <ENTER> to end the path

Camera path complete

svpath

Creates an animated sky_vector path for use with an animated camera path

Syntax is the save as that for campath

target

Creates a target (look_at) point for use with a camera path (not necessary when using a camera point or a target path)

Command:target

Target location: select point or enter coordinates

Target point complete

tarpath

Creates an animated target path

Syntax is the save as that for campath

lights

Creates a light object

Command:lights

Light location: select point or enter coordinates

Light color <White>: type valid color or <ENTER> for White

Falloff distance <None>: select distance or <ENTER> for none

Fade power <None>: select distance or <ENTER> for none

Light source complete—a point is drawn at the light source

photon

Adds a photons block to a solid object for use with Nathan Kopp’s uvPOV (see uvPOV documentation for specifics on photon usage)

Command:photon

Select object to add photons: select solid object

If an invalid object is selected photon aborts

Separation distance <0.02>: select or enter distance—default is always 0.02

Refraction on <Y/N>? : enter Y or N

Reflection on <Y/N>? : enter Y or N

Ignore photons <Y/N>? : enter Y or N

Photons complete

## Modify

### Command syntax

punion

Unions two or more solid primitives

Command:punion

Select 3DSolids to join: select two or more solid objects

Merge complete

pmerge

Merges two or more solid primitives

Similar to punion

psub

Subtracts solid primitives from other solid primitives

Command:psub

Select 3DSolids to subtract from: select one or more solid objects

Select 3DSolids to subtract: select one or more solid objects

Difference complete

pinterf

Creates a solid from the common area of two or more solid primitives

Command:pinterf

Select 1st set of 3DSolids: select one or more solid objects

Select 2nd set of 3DSolids: select one or more solid objects

Intersection complete

pthick

Extrudes 2D lines to 3Dfaces in the U-direction

Command:pinterf

Select objects: select lines to add thickness to

Thickness of lines: select or enter distance

pmove

Substitute command for move for use on solid objects only

Command:pmove

Select object to move: select solid object to move

If an invalid object is selected, pmove aborts

Base point for translation: select point or enter coordinates

Base point for translation: select point or enter coordinates

Second point of displacement: select point or enter coordinates

Translate complete

pmirror

Substitute command for mirror for use on solid objects only

Similar to pmove

pscale

Substitute command for scale for use on solid objects only

Similar to pmove

pcopy

Substitute command for copy for use on solid objects only

Similar to pmove

protate

Substitute command for rotate for use on solid objects only

Similar to pmove

## Other

saveaspov

Shortcut to dxfout command

With saveaspov, you may selectively export objects to a DXF file.  Therefore, you can experiment with various lights, cameras, targets, etc…Also, if you redefine a block in AutoCAD, there is no need to reconvert the entire drawing.  Simply explode the block (after inserting it at 0,0,0) and saveaspov.  Then convert the file to a .inc file by checking Block file only in the executable. You can see the options available with this flexibility.

# AutoCAD2POV.exe

The executable converts DXF files including data added by the LISP file.  The executable has support for all objects mentioned in section 4 as well as blocks (consisting of valid objects), 3Dfaces and 3D Meshes.  The interface is rather simple and intuitive (I would like to think) so I won’t go into much detail.  AutoCAD2POV can be started from the command line with no user interface.  The syntax is:

## Files

AutoCAD2POV creates backups of each file before overwriting them.

The executable creates some or all of the following files:

Myfile.pov—includes myobject and mycamera and global_settings if there are photon blocks

Myfile.inc—declares myobject and mycamera

MyfileT.inc—declares all layers as textures

Myblock.inc—declares myblock for insertion in myobject

Myfilec.path—camera path data

Myfilet.path—target path data

Myfiles.path—sky vector path data

Myfile.ini—.ini file declaring clock and frames for animation

## Blocks

AutoCAD2POV creates a separate .inc file for each block in the drawing.  The files are named according to the block name in AutoCAD.  The blocks appear in myfile.inc as object { myblock }, with the myblock.inc file being included at the beginning of myfile.inc.  When re-converting a DXF file, if the blocks have not changed, you can save time by unchecking Create new block files.

## Layers

MyfileT.inc defines all layers as textures.  All textures are in uppercase, so layer T_STONE13 does not become texture T_Stone13.  AutoCAD2POV defines each texture as a pigment only.  Each pigment will match the layer color if color number is 1-9, otherwise it is white.  If you have a layer named GLASS, it is defined as a material with entries for Mr. Kopp’s uvPOV.  If you are not using uvPOV, you will have to modify this entry or avoid using layers named GLASS.  After you have gone to such hard work perfecting your textures, uncheck the box to Create new texture file when converting the DXF file next time to avoid overwriting the modified file.

## Animation

When converting the DXF file,  .path files and .ini files are only created if a camera path or target path is defined.  If there is both a camera point and a camera path in the DXF file, the camera path will be defined as the camera in POV-Ray and the camera point will be ignored.  This same holds true for target points and paths.  If there is more than one camera/target point/path then the one that appears last in the entity databse will be used for rendering.  See section , saveaspov, for further thoughts on this point.

# Contact the author

You can contact the author by e-mail at cadman@graffiti.net.  If you need to download the full install of AutoCAD2POV or find out any late-breaking information, surf to http://www.cgirealm.com/ccs/acad2pov.html.

# In the future

Should there happen to be enough interest in this package, I will continue to update it as ideas and bugs are submitted (by you).  Better error checking will be a priority.

# Known issues

No big ones at the time of printing, although it’s hard to find beta testers around here.  I’m fairly sure that it won’t erase your hard drive devalue the yen.  You shouldn’t be investing overseas, anyway…

There are some things to keep in mind, though.  If an error occurs inside a custom command (such as the object being on a locked layer, etc…) the wrong object could be wrongly created and/or modified and it won’t show up until the rendering in POV-Ray.  The best way to avoid this is to UNDO one step in AutoCAD after aborting a custom command.

There seems to be inconsistent errors with the camera/target path macro that is inserted at the beginning of myfile.inc.  This is only used when a camera or target path is defined.  Occasionally the parser seems to ignore a couple characters any cause an error.  I haven’t been able to pin down the problem, though, so hopefully it doesn’t happen to you.